Interrupts Tutorial Exercise

It is advised that when you attempt this exercise either during in-class or in tutorial, please work in groups.

## Question 1

The table below shows the Initial State of registers:

| Register | Value |  | Register |  | Value | Register/State | Value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R0 | 10 |  | R8 |  | 18 | xPSR | 0x0100_0000 |
| R1 | 11 |  | R9 |  | 19 | MSP | 0x2000_0FF4 |
| R2 | 12 |  | R10 |  | 20 | PSP | n/a |
| R3 | 13 |  | R11 |  | 21 | PRIMASK | 0x0000_0000 |
| R4 | 14 |  | R12 |  | 22 |  |  |
| R5 | 15 |  | R13 (SP) |  | 0x2000_0FF4 | Mode | Thread |
| R6 | 16 |  | R14 (LR) |  | 0x1000_0000 | Privilege | Privileged |
| R7 | 17 |  | R15 (PC) |  | 0x0000_0220 | Stack | Main |
| Exception No. |  | Value |  |
| 12 |  | 0x0000_1441 |  |
| 13 |  | 0x0000_1211 |  |
| 14 |  | 0x0000_1335 |  |
| 15 |  | 0x0000_1669 |  |
| 16 |  | 0x0000_120D |  |
| 17 |  | 0x0000_1129 |  |
| 18 |  | 0x0000_1351 |  |
| 19 |  | 0x0000_1539 |  |
| 20 |  | 0x0000_1285 |  |
| 21 |  | 0x0000_183D |  |
| 22 |  | 0x0000_1849 |  |
| 23 |  | 0x0000_1661 |  |

The CPU then responds to an IRQ2 exception request.

Write down the values for the registers, after responding to exception request and before executing the first instruction exception handler:

| Register | Value | Register | Value | Register/State | Value |
| --- | --- | --- | --- | --- | --- |
| R0 |  | R8 |  | xPSR |  |
| R1 |  | R9 |  | MSP |  |
| R2 |  | R10 |  | PSP |  |
| R3 |  | R11 |  | PRIMASK |  |
| R4 |  | R12 |  |  |  |
| R5 |  | R13 (SP) |  | Mode |  |
| R6 |  | R14 (LR) |  | Privilege |  |
| R7 |  | R15 (PC) |  | Stack |  |

Also write down the addresses and contents of the stack area.

| Address | Value | Description |
| --- | --- | --- |

## Question 2

What is the return address will an exception handler return to, given the following register and memory contents? Write and explain your logic behind your deduction. 
Assume the instruction for exiting the exception handler is POP {R4,PC}.

## Question 3

Consider a system built on a 48 MHz processor with a main loop and an interrupt which occurs at a 10 kHz frequency. Assume the ISR take 14.9 us to execute and there is a total of 1 us of response and return time overhead.

(a) What percentage of the processor’s time is spent in the ISR and the overhead?

(b) What percentage of the processor’s time is left for the main loop to execute?

(c) If the main loop requires 37 ms of computation to execute one iteration, what is the maximum main loop update rate (in Hz)?

Consider the system above again, but now with the interrupt running at 25 kHz and the ISR taking 34 us to execute.

(d) What percentage of the processor’s time is spent in the ISR and the overhead?

(e) What percentage of the processor’s time is left for the main loop to execute?

(f) If the main loop requires 37 ms of computation to execute one iteration, what is the maximum main loop update rate (in Hz)?

- End of Tutorial Exercise